名古屋市近郊における大気及び環境水中の窒素酸化物と硫黄

サンプル 1

#### 酸化物計測へのフローインジェクション分析法の導入

手嶋　紀雄1，酒井　忠雄1,＊，安田　誠司2，樋口　慶郎3

1愛知工業大学応用化学科：470-0392　愛知県豊田市八草町八千草1247

2(財)東海技術センター：465-0021愛知県名古屋市名東区猪子石2-710

3東京化成工業(株)FIA事業部：114-0003東京都北区豊島6-15-9

#### Application of FIA to the Measurement of NOx and SOx in Air Samples and Environmental Waters at Nagoya and Its Environs

Norio Teshima1, Tadao Sakai1,＊, Seiji Yasuda2 and Keiro Higuchi3

# 1 Department of Applied Chemistry, Aichi Institute of Technology, 1247 Yachigusa, Yakusa-cho,

# Toyota 470-0392, Japan

# 2 Tokai Technology Center, 2-710 Inokoishi, Meitoh-ku, Nagoya-shi, Aichi 465-0021

# 3 FIA Division, Tokyo Kasei Kogyo Co.Ltd., 6-15-9 Toshima, Kita-ku, Tokyo 114-0003

Flow injection analysis (FIA) was applied to simultaneous determination of nitrogen monoxide (NO), nitrogen dioxide (NO2) and sulfur dioxide (SO2) as air pollutants collected by using the passive sampling device. NO and NO2 were determined based on a diazotization/coupling reaction with the copperized cadmium (Cd/Cu) reduction column installed in FIA system. A NOx (NO+NO2) collection filter was coated with PTIO (2-phenyl-4,4,5,5-tetramethylimidazoline-3-oxide-1-oxyl), which oxidized NO to NO2, together with triethanolamine for collecting NO2. However, the residual PTIO that coexisted in the extract for FIA measurement interfered the diazotization/coupling reaction. We found that the interference of the residual PTIO can be completely removed by using the on-line Cd/Cu column in the FIA system. An FIA method using dimethylsulfonazo III-barium chelate was used for SO2 determination. By using these FIA systems, NO, NO2, NOx and SO2 in air samples collected at Nagoya and its environs were monitored, and the relation between the results and weather conditions was considered. On the other hand, the same proposed FIA methods were applied to the monitoring the concentrations of nitrite nitrogen and nitrate nitrogen in river water, pond water, rainwater and seawater, and of sulfate ion in seawater. The proposed FIA systems provide simple and rapid methods with high precision for the determinations of nitrogen oxides and sulfur oxides and thus are useful for the investigation of the movement of these pollutants in air and water samples.

Keywords nitrogen oxides, sulfur oxides, air pollutants analyses, water analyses, FIA

1. 緒　言

窒素酸化物（NOx）及び硫黄酸化物（SOx）は，環境中で種々の化学形態をとりながら循環を繰り返していると考えられ，その動態把握は環境・健康科学的に極めて重要である。大気中へのNOx及びSOxの排出量は，最近になって自動車NOx法等による規制や触媒化学の進歩により，横這い状態といわれている。国や地方自治体は常時監視局を設置し，NOx，SOx等の測定を行っているが，大型装置による定点測定であることから，汚染状況を十分に把握することは困難である。例えば交通量の多い道路周辺では環境基準を超えることもあり，気象条件によっては広域拡散し，間接的に人体の健康に及ぼす影響が懸念される。また近年，地球規模の問題として注目されている酸性雨は，その主な原因物質が硝酸と硫酸であり，それらの前駆物質はNOxとSOxである。したがって，これら酸化物の動態を把握するためには大気だけでなく，河川，池水等の水圏におけるNOx，SOx濃度の監視も必要である。

近年，これらの環境汚染物質を任意の場所を選んでサンプリングすることができる拡散型サンプラー，いわゆるパッシブサンプラーが考案された[1,2]。二酸化窒素（NO2）の個人被ばく（曝）量測定等に利用できるバッジ型のサンプラーも市販されているが，一酸化窒素（NO）とNO2とを同時に測定できるようには設計されていない。平野らは，このサンプラーを改良し，NOとNO2[3]及び二酸化硫黄（SO2）[4,5]を同時に捕集できるパッシブサンプラーを開発した。サンプリング後の濃度測定は，NOxの場合バッチマニュアル式の吸光光度法で行われているが，捕集ろ紙からのNO2–抽出液には未反応のPTIO（2-フェニル-4,4,5,5-テトラメチルイミダゾリン-3-オキシド-1-オキシル）が残留しており，これが測定精度の低下を招いていた。一方，SO2の測定にはイオンクロマトグラフィーが用いられているが，